IoT Speed Camera &

Remote-Controlled Car

Technical Project Report

Embedded Software for the Internet of Things

Group 14

Team Members

| Name | Student ID |
| --- | --- |
| Luciani Stefano | 243851 |
| De Cao Andrea | 244157 |
| Boscardin Denise | 243932 |
| Heenatigala Devmin | 242916 |

Academic Year 2025/2026

# 1. Project Overview

This project was developed as part of the Embedded Software for the Internet of Things course. It consists of two main subsystems: a gesture-controlled remote-controlled (RC) car, and a scaled-down speed camera system. 
Together they form a complete IoT pipeline that spans hardware sensing, wireless communication, embedded control, and web-based data visualization.

The RC car is a custom-built four-wheeled vehicle assembled on a chassis with four 5V DC motors controlled by an L298N H-bridge driver, powered by an Arduino Uno R4 WiFi. It is operated through hand gestures detected by an MPU6050 gyroscope sensor mounted on a separate Arduino Nano controller unit. 
The operator tilts their hand to steer and accelerate the vehicle, with movement commands transmitted wirelessly via nRF24L01 radio modules. 
The speed camera, built around an ESP32 microcontroller, measures the vehicle speed using two VCNL4010 proximity sensors and compares it against a configurable threshold.
Detection results are transmitted to a cloud-hosted Node.js web server and displayed in a real-time dashboard.

# 2. System Architecture

The system is composed of three hardware units and one software layer:

## 2.1 Arduino Nano — Gesture Controller

The controller unit is built around an Arduino Nano paired with an MPU6050 IMU. The MPU6050's Digital Motion Processor (DMP) computes quaternion-based orientation data, which is converted to Yaw, Pitch, and Roll angles. Roll controls left/right steering, Pitch controls forward/backward movement. Angle values are constrained to the range [-50°, +50°] and mapped to a byte (0–254) for compact transmission over nRF24L01 at 250 kbps on channel 108 (outside standard Wi-Fi bands).

## 2.2 Arduino Uno R4 WiFi — Car Receiver

The car unit receives the nRF24L01 packets and uses differential drive logic to compute independent left and right motor speeds. The L298N H-bridge motor driver controls four DC motors. Speed for each side is derived from the received Roll and Pitch values, enabling smooth curved trajectories. The radio operates in RX mode with automatic ACK to confirm packet reception.

## 2.3 ESP32 — Speed Camera Unit

The speed camera uses two VCNL4010 proximity sensors mounted 4 cm apart on separate I2C buses. A finite state machine (FSM) manages the detection pipeline: the system waits for the first sensor to trigger, records a microsecond timestamp, then waits for the second sensor. Speed is calculated as distance divided by elapsed time and compared against the 0.75 m/s threshold.

## 2.4 Node.js Web Server — Data Layer

The backend is a Node.js/Express server deployed on Render. It exposes three REST endpoints:

| Endpoint | Method | Description |
| --- | --- | --- |
| POST /salva | POST | Receives speed and overspeed flag from the ESP32, appends a timestamped row to a CSV log file with an auto-incremented test case ID. |
| GET /dati | GET | Returns the full speed log as a JSON array for table display in the dashboard. |
| GET /data | GET | Returns only the latest reading (speed + sensor flag) for real-time display. |

Data is persisted in a CSV file (speed-log.csv) with the columns: Test-Case-ID, Speed, Sensor-Triggered, Timestamp. 
The server also serves the frontend static files from the public/ directory.

# 3. Firmware Description

## 3.1 Speed Camera FSM (fsm_velox.ino)

The speed camera firmware implements a 4-state FSM:

| State | Description |
| --- | --- |
| WAIT | Idle state. Updates the OLED display and transitions to IR1_STATE. |
| IR1_STATE | Polls BAR1 proximity. When proximity exceeds the calibrated threshold, records tempo1 (microseconds) and transitions to IR2_STATE. |
| IR2_STATE | Polls BAR2 proximity. Enforces a 3-second timeout: if the vehicle does not pass within the limit, resets to WAIT. On trigger, records tempo2. |
| COMPUTATION | Calculates speed = (4 cm / delta_t) converted to m/s. Sets overSpeed flag, updates OLED, saves to flash, and sends an HTTPS POST to the server. |

Sensor calibration is performed at startup: 20 proximity readings are averaged per sensor, and a fixed offset (SOGLIA_OFFSET = 50) is added to define the detection threshold, accounting for ambient IR noise.

## 3.2 Gesture Transmitter (nRF24L01_TX.ino)

The transmitter firmware runs on Arduino Nano. The MPU6050 DMP is initialized with 6-cycle accelerometer and gyroscope calibration for accurate zero-gravity offset. In the main loop, each FIFO packet from the DMP is read, quaternions are extracted, gravity is computed, and Euler angles (Yaw/Pitch/Roll) are derived. Angles are clamped and mapped to bytes, packed into a 3-byte DataPacket struct, and transmitted via radio.write(). The nRF24L01 is configured with 5 retries at 250 us intervals to handle packet loss.

## 3.3 Car Receiver (nRF24L01_RX.ino)

The receiver firmware runs on Arduino Uno R4 WiFi. The radio is set to RX mode on the same channel and address as the transmitter. On each received packet, the movimento() function computes differential motor speeds: moveX from Roll (left/right bias) and moveY from Pitch (forward/backward velocity). The constrain() function ensures PWM values stay within the 0–255 range, enabling smooth turning at variable speeds.

# 4. Web Server & Dashboard

## 4.1 Backend (server.js)

The server is implemented with Express.js and reads/writes a local CSV file for data persistence. On startup, it reads the existing CSV to restore the currentTestCaseId counter, ensuring unique IDs across server restarts. The POST /salva endpoint includes a basic duplicate-detection check comparing the last CSV row against the incoming data before appending.

## 4.2 Frontend Website (index.html)

The project website serves as the main entry point, providing a project overview, an image slider with hardware schematics, a system architecture section, key features, a how-it-works walkthrough, an embedded YouTube demonstration, and links to firmware and documentation.

## 4.3 Real-Time Dashboard (DashBoard.html)

The dashboard polls the GET /data endpoint at regular intervals to display the latest vehicle speed and overspeed status. It also fetches the full log from GET /dati to populate a historical data table. The dashboard is accessible at iot-project-group-14.onrender.com.

# 5. Communication Protocol

| Link | Protocol | Parameters | Notes |
| --- | --- | --- | --- |
| Controller > Car | nRF24L01 (SPI) | 250 kbps, channel 108, PA_MIN | 3-byte struct: Roll, Pitch, Yaw (0–254 each) |
| ESP32 > Server | HTTPS / REST | POST /salva, JSON body | WiFiClientSecure, certificate check disabled |
| Server > Dashboard | HTTP / REST | GET /data, GET /dati | JSON response, polled by frontend JS |
| I2C (ESP32) | I2C @ 100 kHz | Two buses: pins 21/22 and 18/19 | VCNL4010 on both buses, OLED on bus 1 |
| I2C (Nano) | I2C @ 400 kHz | Fast Mode, 3 ms timeout | MPU6050 DMP, EMI protection via timeout |

# 6. Hardware Components

## 6.1 RC Car

| Component | Purpose |
| --- | --- |
| Arduino Uno R4 WiFi | Main car controller, motor driver interface |
| Arduino Nano | Gesture controller unit |
| MPU6050 | Gyroscope/accelerometer for gesture detection |
| nRF24L01+ PA LNA (x2) | Wireless communication between controller and car |
| L298N Motor Driver | Dual H-bridge for 4 DC motors |
| 4x 5V DC Motors + Tires | Propulsion system |
| Power Bank 5000/10400 mAh | Power supply for electronics and motors |

## 6.2 Speed Camera

| Component | Purpose |
| --- | --- |
| ESP32 | Main speed camera controller, WiFi connectivity |
| VCNL4010 (x2) | Proximity sensors for speed measurement (4 cm apart) |
| SSD1306 OLED Display | Real-time status and speed display |
| Breadboard & Jumper Wires | Circuit connections |

# 7. Software Requirements

| Tool / Library | Version | Purpose |
| --- | --- | --- |
| Arduino IDE | 2.x | Firmware development and upload |
| RF24 | Latest | nRF24L01 radio communication |
| MPU6050 / I2Cdev | Latest | Gyroscope DMP processing |
| Adafruit VCNL4010 | Latest | Proximity sensor driver |
| Adafruit SSD1306 | Latest | OLED display driver |
| Node.js / Express | LTS | Web server backend |
| Render | — | Cloud deployment platform |

# 8. Team Members & Contributions

## Luciani Stefano

Responsible for the hardware design, component wiring and the development of the control and communication software for RC car.
Responsible for the development of the finite state machine of the velox and its hardware design.

## De Cao Andrea

Responsible for the overall project structure and architecture. Co-developed the control and communication software for the RC car, developed a Node.js backend exposing a REST API to receive and serve real-time sensor data, and a frontend Dashboard displaying live vehicle data. Handled the frontend–backend integration and deployment on Render.

## Boscardin Denise

Responsible for the speed camera firmware, 3D printing of physical enclosures and mechanical parts, README documentation, and video editing of the project demonstration.

## Heenatigala Devmin

Responsible for the speed camera code and code polishing.

# 9. Deployment & Setup

## 9.1 Web Server

To run the web server locally:

cd src/web_server npm install npm start # Server starts at http://localhost:3000

The server is also deployed on Render at:

https://iot-project-group-14.onrender.com/

## 9.2 Firmware Upload Order

- Upload fsm_velox.ino to ESP32 (speed camera).

- Upload nRF24L01_RX.ino to Arduino Uno R4 WiFi (car receiver).

- Upload nRF24L01_TX.ino to Arduino Nano (gesture controller).

- Keep the MPU6050 flat during Nano startup for calibration.

- Power on the car, then the speed camera.

- Open the web dashboard to monitor speed data.

# 10. Links & Resources

| Resource | URL |
| --- | --- |
| Web Dashboard | https://iot-project-group-14.onrender.com/ |
| GitHub Repository | https://github.com/AndreaDeCao/IoT_project |
| YouTube Demo | https://www.youtube.com/watch?v=iuvEgluySTU |